WEAVING YARN CALCULATION

Complete Formula & Calculation Reference Guide

# 1. Default Input Parameters

These are fixed baseline values used throughout all calculations. They can be adjusted based on count and quality requirements.

| Parameter | Default Value | Purpose |
| --- | --- | --- |
| Grey Reed Add | 4 | Extra reed allowance added for grey (unfinished) fabric |
| Reed Less | 8 | Reed reduction based on yarn count & quality |
| Fabric Width Reed Add | 4 | Additional reed allowance for fabric width calculation |
| Pick Less | 4 | Reduction in picks for on-loom setting |
| Warp Yarn Calc Parameter | 1690 | Standard conversion constant for warp yarn KGS calculation |
| Dyeing Loss % | 5% | Default yarn loss during dyeing process |

# 2. Order & Meterage Calculation

This section calculates the total metres of warp and weft yarn required based on the buyer's order quantity.

| Field | Formula | Example Value |
| --- | --- | --- |
| Order Mtr | Buyer input | 60 mtr |
| Excess Mtr | Buffer quantity (optional) | - |
| Total Order Mtr | Order Mtr + Excess Mtr | 60 mtr |
| Warp Mtr (Bottom) | Total Order Mtr x (1 + Crimp% + Shrinkage%) | 120 mtr |
| Prod Mtr (Weft) | Production metres for weft calculation | 90 mtr |
| Top Warp Mtr | Same logic as Bottom Warp Mtr | 120 mtr |

# 3. Crimp % and Shrinkage %

These percentages measure how much the fabric contracts during weaving and processing. They are critical for accurate yarn ordering.

## 3.1 Bottom Warp Crimp %

Measures how much the warp yarn 'crimps' as it interlaces with the weft thread during weaving.

Bottom Warp Crimp % = (Warp Mtr - Fabric Mtr) / Fabric Mtr x 100

## 3.2 Shrinkage %

Measures how much the fabric shrinks after wet processing (washing, finishing, etc.).

Shrinkage % = (Fabric Mtr - Total Order Mtr) / Total Order Mtr x 100

## 3.3 Top Warp Crimp %

Same as bottom warp crimp, but calculated for the top beam in double-beam looms.

Top Warp Crimp % = (Top Warp Mtr - Fabric Mtr) / Fabric Mtr x 100

# 4. Reed Calculations

The reed is the comb-like device on the loom that spaces and separates the warp threads. Reed calculations determine the correct loom settings.

| Field | Formula | Example Value |
| --- | --- | --- |
| Finished Reed | Input (based on fabric spec) | 80 |
| Grey Reed | Onloom Reed + Grey Reed Add (4) | 76 |
| Onloom Reed | Finished Reed - Reed Less (8) | 72 |
| Finished Pick | Input (based on fabric spec) | 72 |
| Onloom Pick | Finished Pick - Pick Less (4) | 68 |
| Total Ends (Manual) | Finished Reed x Finished Width (inch) | 4,104 |
| Total Ends (Auto) | Calculated automatically by formula | 4,560 |
| Reed Space | Total Ends / Onloom Reed | 63.33 inches |
| Fabric Width (Grey) | Total Ends / (Onloom Reed + Grey Reed Add) | 60 inches |

# 5. Design Ends Finding

Used to determine how many warp ends fit within a specific measurement in millimetres. Useful for design repeat planning.

Key conversions:

- 1 Inch = 25.4 MM

- 1 MTR = 39.37 Inches

Required Ends = (Required MM / 25.4) x Finished Reed

Example: Required MM = 3 → Required Ends = (3 / 25.4) x 80 = 10 ends

# 6. Reed / Dent Calculation Table

This table shows how many ends (threads) pass through each dent of the reed, and the resulting EPI (Ends Per Inch).

| Reed Type | Dents per Inch | Ends per Dent | Total EPI |
| --- | --- | --- | --- |
| R3 | 1.5 | 64 | 96 |
| R4 | 2.0 | 72 | 144 |

EPI (Ends Per Inch) = Reed dents x Ends per dent — controls warp thread density in the fabric.

# 7. Warp Yarn Calculation (KGS)

This section calculates how many kilograms of warp yarn are needed for each colour, including a buffer for dyeing losses.

## 7.1 Required KGS Formula

REQ KGS = Ends x 1.094 / 1848 / Warp Count x Warp Mtr

Where:

- 1.094 = Conversion factor (yards to metres)

- 1848 = Hanks per kg constant for yarn count system

- Warp Count = Yarn count (e.g. 20s, 40s)

- Warp Mtr = Total warp metres required

## 7.2 With Dyeing Loss

With Dyeing Loss KGS = REQ KGS / (1 - Dyeing Loss %)

Extra yarn is ordered to compensate for losses during the dyeing process (default 5%).

## 7.3 Example (CREAM colour)

| Colour | Count | Ends | REQ KGS | With Dyeing Loss | Rounded |
| --- | --- | --- | --- | --- | --- |
| CREAM | 20s | 1,235 | 4.38 | 4.60 | 5 KGS |
| L.GREY | 20s | 298 | 1.06 | 1.11 | 1 KGS |
| D.GREY | 20s | 885 | 3.14 | 3.30 | 3 KGS |
| BLACK | 20s | 546 | 1.94 | 2.04 | 2 KGS |

# 8. Weft Yarn Calculation (KGS)

Calculates the weft (filling thread) yarn needed per colour for the full order quantity.

## 8.1 Weft Ends Calculation

Weft Ends = Onloom Pick x (Reed Space + 3 inches selvage)

3 inches is added as selvage allowance on both sides of the fabric.

## 8.2 KGS per Colour

Total per colour = (Colour Ends / Total Weft Ends) x Total Weft Length

Dyeing loss is then added in the same way as warp yarn:

With Dyeing Loss KGS = REQ KGS / (1 - Dyeing Loss %)

## 8.3 Example

| Colour | Count | Weft Ends | REQ KGS | With Dyeing Loss | Rounded |
| --- | --- | --- | --- | --- | --- |
| CREAM | 20s | 70 | 4.69 | 4.93 | 5 KGS |
| L.GREY | 20s | 14 | 0.94 | 0.99 | 1 KGS |
| D.GREY | 20s | 48 | 3.22 | 3.38 | 3 KGS |
| BLACK | 20s | 30 | 2.01 | 2.11 | 2 KGS |

## 8.4 Known Implementation Issue: Total Weft Ends Inflated

In the Design Entry backend, the Weft Subtotal has been found to disagree with the formula in Section 8.1 above. All colours in the affected design were inflated by the same ~1.058-1.059x factor, which is the signature of a single shared value being wrong (Total Weft Ends), not a per-colour data-entry problem.

Worked check (example order: On-loom Pick 52, Total Ends 3,808, On-loom Reed 60):

| Quantity | Correct (per 8.1) | Seen in app |
| --- | --- | --- |
| Reed Space = Total Ends / On-loom Reed | 3,808 / 60 = 63.47" | - |
| Total Weft Ends = On-loom Pick x (Reed Space + 3) | 52 x 66.47 = 3,456 | 3,659 (+5.9%) |

Back-solving the app figure (3,659 / 52 = 70.37") shows an effective "Reed Space + 3" about 3.9" too high. Because the Warp side does not use Reed Space or On-loom Pick at all, and Warp reconciles correctly, the bug is isolated to the Weft "Total Weft Ends" function only.

Checklist to fix / verify:

1. Confirm Reed Space is recalculated for the CURRENT design (Total Ends / On-loom Reed of this design), not read from a previous design's cached state.

2. Confirm the On-loom Reed used as the denominator is this design's On-loom Reed, not Grey Reed or a hardcoded default.

3. Confirm the +3" selvage constant is added exactly once.

4. Confirm On-loom Pick feeding this formula matches the value shown in the design's own On-loom Pick field.

5. After the fix, Total Weft Ends should equal On-loom Pick x (Reed Space + 3) exactly, and the Weft Subtotal "Total End" column should sum to that same number.

# 9. Dyeing Loss & Issued KGS Tracking

This section tracks the actual yarn issued to the dyehouse versus what was received back, to monitor real dyeing losses.

| Field | Formula | Example Value |
| --- | --- | --- |
| Total Issued KGS | Total yarn sent to dyehouse | 192.56 KGS |
| Received KGS | Yarn received after dyeing | 186.70 KGS |
| Balance KGS | Issued - Received | 5.86 KGS |
| Dyeing Loss % | (Issued - Received) / Issued x 100 | ~3.04% |

# 10. Weaving Crimp & Processing Shrinkage

These calculations track fabric dimensional changes through the entire production process.

## 10.1 Weaving Crimp %

Measures fabric length loss during the actual weaving process.

Weaving Crimp % = (Warp Mtr - Weav Mtr) / Warp Mtr x 100

## 10.2 Processing Shrinkage %

Measures fabric length loss during wet processing (washing, finishing).

Processing Shrinkage % = (Grey Mtr - Processed Mtr) / Grey Mtr x 100

## 10.3 Example Values

| Stage | Metres | Loss |
| --- | --- | --- |
| Grey (after weaving) | 3,689.7 mtr | - |
| Processed (after finishing) | 3,364.2 mtr | 325.5 mtr |
| Processing Shrinkage % | - | 8.82% |

# 11. GSM and GLM Calculation

GSM (Grams per Square Metre) and GLM (Grams per Linear Metre) measure the weight of the fabric.

## 11.1 GSM Formula

GSM = ((EPI / Warp Count) + (PPI / Weft Count)) x 25.4

Where:

- EPI = Ends Per Inch (warp thread density)

- PPI = Picks Per Inch (weft thread density)

- 25.4 = Conversion factor for metric system

## 11.2 GLM Formula

GLM = GSM x (Fabric Width / 39.37)

GLM gives the weight of one metre length of fabric at the full fabric width.

# 12. GST Calculation (Sheet 2)

Calculates Indian Goods & Services Tax for textile invoicing.

| GST Type | Applicability | Rate | Components |
| --- | --- | --- | --- |
| Intra-State GST | Within same state | 5% total | SGST 2.5% + CGST 2.5% |
| Inter-State GST | Between states | 18% | IGST 18% |

SGST = Taxable Value x 2.5%

CGST = Taxable Value x 2.5%

IGST = Taxable Value x 18%

Grand Total = Taxable Value + Tax Amount

# 13. Resultant Count Calculation

When yarn is plied (twisted together), the resultant count determines the effective yarn thickness.

Resultant Count = Single Yarn Count / Number of Plies

| Yarn | Single Count | Plies | Resultant Count |
| --- | --- | --- | --- |
| 2/60s | 60s | 2 | 30s |
| 2/40s | 40s | 2 | 20s |

# 14. Bale Packing Tolerance

A standard tolerance is applied to dispatch quantities to account for small variations in fabric length during rolling and packing.

| Parameter | Value |
| --- | --- |
| Default Tolerance | +/- 8% |
| Tolerance Calculation | Order Mtr x Tolerance % |
| Max Dispatch Mtr | Order Mtr + (Order Mtr x 8%) |
| Min Dispatch Mtr | Order Mtr - (Order Mtr x 8%) |

# 15. Key Terms Reference

| Term | Full Form | Meaning |
| --- | --- | --- |
| EPI | Ends Per Inch | Number of warp threads per inch - controls warp density |
| PPI | Picks Per Inch | Number of weft threads per inch - controls weft density |
| GSM | Grams Per Square Metre | Fabric weight measurement |
| GLM | Grams Per Linear Metre | Weight of one metre length of fabric |
| Reed | N/A | Comb-like device on loom that spaces warp threads |
| Crimp | N/A | Waviness of yarn as it interlaces - causes length loss |
| Selvage | N/A | The woven edges on both sides of the fabric (3 inches added) |
| Beam | N/A | Roll on which warp yarn is wound before weaving |
| Grey Fabric | N/A | Fabric as it comes off the loom, before processing |
| Dyeing Loss | N/A | Yarn weight lost during the dyeing process |
| SGST/CGST | State/Central GST | Intra-state tax split equally between state and center |
| IGST | Integrated GST | Single tax for inter-state transactions |

Weaving Yarn Calculation Reference | All formulas based on ORIGINAL sheet data