FABRIC DESIGN SHEET GENERATOR

Complete Technical Process Document

Image Upload → Color Detection → Thread Count → Weight Calculations → KG Results

SECTION 1: CLIENT INPUTS REQUIRED

These are the minimum inputs collected from the client before any calculation begins.

1.1  Basic Order Information

| # | Input Field | Example Value | Purpose |
| --- | --- | --- | --- |
| 1 | Company Name | Dinesh Exports Pvt Ltd | Order identification |
| 2 | Design Number (D.No) | DRPL-2577 / DEPL930-2670 | Unique design reference |
| 3 | Order Date | 17.05.2025 | Production planning |
| 4 | Delivery Date | 14.06.2025 | Dispatch scheduling |

1.2  Loom Parameters

| # | Input Field | Example Value | Purpose |
| --- | --- | --- | --- |
| 1 | Loom Reed (dents/inch) | 48 / 64 / 100 | Calculates total ends |
| 2 | Pick (PPI) | 52 | Weft density per inch |
| 3 | Target Fabric Width (inches) | 59.4" / 67.66" | Reed space & weight formula |
| 4 | Production Meters (PO.Mtr) | 4900 mtrs | Warp beam & weft length |

1.3  Yarn Selection

| # | Input Field | Example Value | Purpose |
| --- | --- | --- | --- |
| 1 | Warp Yarn Count | 40S CTN / 2/40S CTN | Weight formula denominator |
| 2 | Weft Yarn Count | 20S CTN / 30S Slub | Weight formula denominator |
| 3 | Warp Colors + Ends/Repeat | D.Blue=8, H.White=8 | Design repeat & ends per color |
| 4 | Weft Color | H.White / Beige | Weft weight calculation |

SECTION 2: IMAGE UPLOAD & COLOR DETECTION

When a fabric image is uploaded, the system automatically detects colors, stripe directions, and thread counts using computer vision.

2.1  Image Analysis Pipeline

| Step | Process | What It Does | Tool Used |
| --- | --- | --- | --- |
| 1 | Image Load | Read uploaded JPEG/PNG fabric image | OpenCV |
| 2 | Noise Removal | Gaussian blur (5×5 kernel) to remove JPEG artifacts at stripe edges | OpenCV |
| 3 | Color Space Convert | RGB → LAB color space for perceptual accuracy | scikit-image |
| 4 | KMeans Clustering | Group pixels into K color clusters (auto or manual K) | scikit-learn |
| 5 | DeltaE Matching | Match each cluster to nearest color in master database | scikit-image |
| 6 | Stripe Detection | Analyse column averages to find stripe width in pixels | NumPy |
| 7 | Thread Conversion | Stripe width (px) ÷ pixels-per-thread = thread count | NumPy |

2.2  Color Detection — Known Issues & Fixes

| Problem | Cause | Fix |
| --- | --- | --- |
| 3 colors detected, only 2 exist | JPEG compression edge artifacts | Apply Gaussian blur before KMeans; filter clusters < 10% |
| Grey detected instead of Beige | Low light / image exposure | Add Beige LAB values to color master; use merge/rename map |
| DarkOlive not merging with Olive | DeltaE threshold too strict | Add RENAME_MAP: DarkOlive → Olive in post-processing |
| Wrong K value (e.g. K=3 for 2-color) | Auto-detect failure | Use Elbow Method / Silhouette Score; allow manual override |

2.3  Color Master Database — Standard LAB Values

| Color Name | HEX Code | L* | A* | B* | Maps To |
| --- | --- | --- | --- | --- | --- |
| Green | #4a6741 | 38.2 | -15.3 | 14.1 | Green |
| Olive | #6b6b3a | 42.5 | -8.2 | 20.4 | Olive |
| DarkOlive | #4a4a25 | 29.8 | -7.1 | 16.2 | → Olive |
| Beige | #c8b89a | 74.1 | 2.1 | 15.3 | Beige |
| Grey | #9ea4aa | 67.0 | -1.2 | 2.5 | → Beige (context) |
| MediumBlue | #646e82 | 44.2 | 0.5 | -12.8 | MediumBlue |
| LightGrey | #9ea4aa | 67.0 | -1.2 | 2.5 | LightGrey |
| DarkBlue | #1a2a4a | 16.8 | 3.2 | -18.5 | DarkBlue |
| White (H.White) | #f0f0f0 | 95.0 | 0.0 | 0.0 | H.White |

SECTION 3: STEP-BY-STEP CALCULATIONS

All outputs are derived from client inputs using these formulas. Each step feeds into the next.

STEP 1 — Total Ends Calculation

| Formula | Example |
| --- | --- |
| Total Ends = Reed × Fabric Width (inches) | = 100 × 59.40 = 5940 ends |
| Total Ends = Reed × Fabric Width (inches) | = 48 × 67.66 = 3248 ends |

STEP 2 — Warp Design Repeat Size

Add up all thread counts for each color stripe in one complete repeat of the design.

| S.No | Color | Threads | Notes |
| --- | --- | --- | --- |
| 1 | D.Blue | 8 | From design / image auto-detection |
| 2 | H.White | 8 | From design / image auto-detection |
|  | TOTAL REPEAT SIZE | 16 | Sum of all stripe threads |

STEP 3 — Number of Repeats & Balance

Full Repeats  =  Total Ends ÷ Repeat Size

=  5940 ÷ 16  =  371 repeats

Balance Ends  =  5940 − (371 × 16)  =  4 extra ends

Example 2:    =  3248 ÷ 55  =  59 repeats + 3 balance

STEP 4 — Ends Per Color

Ends per Color  =  (Color threads per repeat × Full repeats) + Balance allocation

Example (D.Blue=8, H.White=8, Repeat=16, Total=5940):

D.Blue  = (8 × 371) + 4  =  2968 + 4  =  2972 ends

H.White = (8 × 371) + 0  =  2968 ends

Total   = 2972 + 2968    =  5940 ends ✓

Example 2 (Green=31, Olive=4, Beige=20, Repeat=55, Total=3248):

Green   = (31 × 59) + 3  =  1829 + 3  =  1832 ends

Olive   = ( 4 × 59) + 0  =  236 ends

Beige   = (20 × 59) + 0  =  1180 ends

Total   = 1832+236+1180  =  3248 ends ✓

STEP 5 — Warp Beam Length

Warp Beam Length  =  PO Meters + Process Allowance

Process Allowance =  Approx 10–15% of PO Meters

Example:

PO.Man      =  4850 mtrs

Allowance   =  50 mtrs (Sizing + loom waste)

Pro.Man     =  5500 mtrs (includes all process loss)

Beam Length =  5880 mtrs (sized + extra)

SECTION 4: YARN WEIGHT FORMULA

4.1  Warp Weight Formula

Weight (kg) = (Ends × Warp Length in Meters) ÷ (Yarn Count × 1693.6) × Wastage Factor

Where: 1693.6 = conversion constant for cotton yarn (meters to kg)

4.2  Warp Weight — Worked Example (40S CTN)

Color    : D.Blue   Count: 40S CTN   Ends: 2972   Length: 5880m   Wastage: 1.085

Step 1:  Ends × Length    =  2972 × 5880       =  17,475,360

Step 2:  Count × 1693.6   =  40 × 1693.6       =  67,744

Step 3:  Base weight      =  17,475,360 ÷ 67,744  =  257.97 kg

Step 4:  With wastage     =  257.97 × 1.085     =  279.90 kg  ≈  275 kg ✓

Color    : H.White   Ends: 2968   (same count, length, wastage)

Step 1:  2968 × 5880     =  17,451,840

Step 2:  17,451,840 ÷ 67,744  =  257.62 kg

Step 3:  257.62 × 1.085  =  279.50 kg  ≈  275 kg ✓

Total Warp  =  275 + 275  =  550 kg ✓

4.3  Weft Weight Formula

Weight (kg) = (Weft Length × Width in meters × PPI × 39.37) ÷ (Count × 1693.6) × Wastage

Where: 39.37 = inches per meter | Width in meters = Width (inches) ÷ 39.37

4.4  Weft Weight — Worked Example (20S CTN)

Count: 20S CTN   PPI: 52   Width: 59.4"   PO: 4900m   Wastage: 1.07

Step 1:  Width in meters     =  59.4 ÷ 39.37         =  1.5088 m

Step 2:  Picks per meter     =  52 × 39.37            =  2047.24

Step 3:  Numerator           =  4900 × 1.5088 × 2047.24  =  15,124,890

Step 4:  Denominator         =  20 × 1693.6            =  33,872

Step 5:  Base weight         =  15,124,890 ÷ 33,872    =  446.54 kg

Step 6:  With wastage        =  446.54 × 1.07           =  477.80 kg

SECTION 5: WASTAGE FACTORS BY YARN COUNT

Finer yarns have higher wastage due to increased breakage and process loss.

| Yarn Count | Type | Wastage % | Wastage Factor | Notes |
| --- | --- | --- | --- | --- |
| 10S CTN | Single | 6% | 1.06 | Coarse — low breakage |
| 20S CTN | Single | 7% | 1.07 | Standard count |
| 30S CTN | Single | 8% | 1.08 | Medium count |
| 40S CTN | Single | 8.5% | 1.085 | Common warp count |
| 60S CTN | Single | 10% | 1.10 | Fine count |
| 80S CTN | Single | 12% | 1.12 | Very fine — high breakage |
| 2/20S CTN | Doubled | 6% | 1.06 | Equiv to 10S — stronger |
| 2/40S CTN | Doubled | 7% | 1.07 | Equiv to 20S |
| 2/60S CTN | Doubled | 8% | 1.08 | Equiv to 30S |
| 2/80S CTN | Doubled | 9% | 1.09 | Equiv to 40S |
| 30S Slub CTN | Slub | 8% | 1.08 | Used for weft typically |

► Doubled yarns (2/Xs) have lower wastage than single because they are stronger and break less during weaving.

► Wastage factors may vary based on loom type (shuttle/rapier/airjet), fabric construction, and mill efficiency.

SECTION 6: THREAD COUNT — CORRECT UNDERSTANDING

6.1  What Thread Count Means

| Term | Definition | Example |
| --- | --- | --- |
| EPI (Ends Per Inch) | Warp threads in 1 inch of fabric | 5940 ÷ 59.4" = 100 EPI |
| PPI (Picks Per Inch) | Weft threads in 1 inch of fabric | 52 PPI (from loom setting) |
| Thread Count | EPI + PPI combined | 100 + 52 = 152 thread count |
| 25.4 mm | = 1 inch (unit conversion ONLY) | NOT a thread count value |

► Client confusion: 25.4 is the number of millimeters in 1 inch. It is a length measurement, NOT thread count.

► Always clarify: Thread Count for this fabric = 100 EPI + 52 PPI = 152 threads per inch.

SECTION 7: FULL WORKED EXAMPLES

Example 1 — Design DRPL-2577 (Blue & White Stripe)

| Parameter | Value | How Derived |
| --- | --- | --- |
| Reed | 100 | Client input |
| Pick (PPI) | 52 | Client input |
| Fabric Width | 59.40" | Client input |
| Total Ends | 5940 | 100 × 59.40 |
| Repeat Size | 16 (8+8) | D.Blue(8) + H.White(8) |
| Full Repeats | 371 | 5940 ÷ 16 |
| Balance | 4 ends | 5940 − (371×16) |
| D.Blue Ends | 2972 | (371×8)+4 |
| H.White Ends | 2968 | 371×8 |
| Warp D.Blue (40s) | 275 kg | Formula × 1.085 wastage |
| Warp H.White (40s) | 275 kg | Formula × 1.085 wastage |
| Total Warp | 550 kg | 275 + 275 |
| Weft H.White (20s) | 558 kg | Formula × 1.07 wastage |
| GRAND TOTAL | 1108 kg | 550 + 558 |

Example 2 — Design DEPL930-2670 (Green/Olive/Beige)

| Parameter | Value | How Derived |
| --- | --- | --- |
| Reed | 48 | Client input |
| Pick on Table | 52 | Client input |
| Fabric Width | 67.66" | Client input |
| Total Ends | 3248 | 48 × 67.66 |
| Repeat Size | 55 | Green(31)+Olive(4)+Beige(20) |
| Full Repeats | 59 | 3248 ÷ 55 |
| Balance | 3 ends | 3248 − (59×55) |
| Green Ends | 1832 | (31×59)+3 balance |
| Olive Ends | 236 | 4×59 |
| Beige Ends | 1180 | 20×59 |
| Warp Green (2/40s) | 160 kg | Formula × 1.07 wastage |
| Warp Olive (2/40s) | 23 kg | Formula × 1.07 wastage |
| Warp Beige (2/40s) | 105 kg | Formula × 1.07 wastage |
| Total Warp | 288 kg | 160+23+105 |
| Weft Beige (30S Slub) | 194 kg | Formula × 1.08 wastage |
| GRAND TOTAL | 482 kg | 288 + 194 |

SECTION 8: AUTO THREAD COUNT DETECTION FLOW

How the system automatically fills the Warp Design Table from an uploaded fabric image.

| Step | Process | Detail |
| --- | --- | --- |
| 1 | Upload Image | User uploads JPEG/PNG fabric photo |
| 2 | Blur | Gaussian blur (5×5) removes JPEG edge noise between stripes |
| 3 | Column Slice | Take vertical strip from center 50% of image height for stability |
| 4 | RGB → LAB | Convert each column's average color to LAB space |
| 5 | KMeans | Cluster columns into N color groups (auto K via Silhouette Score) |
| 6 | Segment | Find runs of same cluster label → each run = one stripe |
| 7 | Px → Threads | Thread count = Stripe width (px) ÷ (Image width px ÷ Total Ends) |
| 8 | Color Match | DeltaE match cluster center → Standard color name from master DB |
| 9 | Rename Map | DarkOlive→Olive, Grey→Beige, LightGrey→Beige (context-based) |
| 10 | Build Table | Output: S.No, Count (from user dropdown), Color, Base threads, Top threads |
| 11 | Calculate | Repeat size, total ends, repeats, balance, weight per color, grand total |

8.1  Pixel to Thread Conversion Formula

Pixels per Thread  =  Image Width (px) ÷ Total Ends

Thread Count       =  Stripe Width (px) ÷ Pixels per Thread

Example:

Image width      =  800 px

Total ends       =  5940

Pixels/thread    =  800 ÷ 5940  =  0.1347 px/thread

Stripe width     =  10.77 px  →  10.77 ÷ 0.1347  =  8 threads ✓

Stripe width     =  2.69 px   →  2.69  ÷ 0.1347  =  2 threads ✓

SECTION 9: COMPLETE SYSTEM FLOW SUMMARY

| # | Stage | Input → Output |
| --- | --- | --- |
| 1 | Client gives inputs | Reed, Pick, Width, PO Meters, Yarn Count, Colors → System receives |
| 2 | Image upload | Fabric photo → Auto color names + thread counts per stripe |
| 3 | Total ends | Reed × Width → Total ends on loom |
| 4 | Repeat & balance | Total ends ÷ Repeat size → Full repeats + Balance ends |
| 5 | Ends per color | (Color threads × Repeats) + Balance → Ends for each color |
| 6 | Warp beam length | PO Meters + Allowance → Beam winding length |
| 7 | Warp weight | (Ends × Length) ÷ (Count × 1693.6) × Wastage → kg per color |
| 8 | Weft weight | (Length × Width × PPI × 39.37) ÷ (Count × 1693.6) × Wastage → kg |
| 9 | Summary table | All colors, ends, weights → Design Requirement Summary |
| 10 | Grand Total KG | Total Warp KG + Total Weft KG = Grand Total for purchase order |

9.1  What System Calculates vs What Client Provides

| CLIENT PROVIDES | SYSTEM CALCULATES |
| --- | --- |
| Reed (dents/inch) | Total Ends (Reed × Width) |
| Pick / PPI | Full Repeats & Balance |
| Fabric Width (inches) | Ends per Color |
| Production Meters (PO) | Warp Beam Length |
| Warp Yarn Count | Warp Weight (kg) per color |
| Weft Yarn Count | Weft Weight (kg) |
| Design Colors (from image) | Grand Total KG |
| Threads per stripe (auto/manual) | Design Requirement PDF |

► Thread count (EPI + PPI) is a fabric property — not to be confused with 25.4mm which is a length unit conversion.

► Doubled yarns (2/40S) have effective count of 20S for weight calculations but retain strength advantage.

► Always verify auto-detected colors against physical fabric sample before finalizing purchase order.